Goal:

Enable real-time identification and prediction of high-risk batches in manufacturing using ML-driven analysis of related traceability and/or characteristics (machine parameters).

Questions that should be answered:

- What value does this provide for our customers?

- Better quality control (more real time defect prediction)

- Cost reduction due to predicting defects before the final product (less scrap and rework)

- Root cause and Pattern Identification

- How well does this technique generalize? Is it a plug-and-play feature that will work for most / every customer, or does it require special training on their data?

It should be plug and play. We can test with multiple algos and auto select the best (have done before in a different feature request.

- How do we determine if the technique is more effective than traditional approaches?

It should be better than just relying on inspections. It would work in tandem with inspections for real time defect detection. With inspections we can verify if the algorithm is working fine.

- How will our customers interact with the technique? Does it happen automatically? What setup is required?

Clean and complete data is required (what GainSeeker does best!). Machine parameters recorded as trace with data are very important (this is where the issue lies since not all customers record it). Usually what trace we record leads only to defect correlation and not causation. Machine parameters could have high rate/probability of causation.

- How much data is required for the technique to work well? Does it require "Big Data" or can it work well with sampling?

It surely would require a few years’ worth of data (1 year minimum) for highly accurate results.

- How much advanced knowledge is required for our customers to find it useful? Is it as simple as clicking a button, or does it require advanced setup and interpretation?

No advanced knowledge required.

- What infrastructure is required on the customers' end? Does it require special sensors and collection?

They would need to record machine parameters with their measurement data (probably as trace) fields. It would be awesome if the parameters come from PLCs rather than operator entry

- Is the learning supervised? Unsupervised? Can it be run automatically in the background or does the customer need to select specific data for training?

Supervised learning. It should run automatically in the background.

- How much will this cost for us to implement? Does it require expensive resources (e.g. GPUs) or lots of customization for every implementation?

Minimal costs are other than the processing power requirements for prediction.

Code explanation –

import streamlit as st

import pandas as pd

import numpy as np

from sklearn.ensemble import RandomForestClassifier

from sklearn.model_selection import train_test_split

from sklearn.metrics import classification_report

# Title

st.title("Batch Defect Prediction Dashboard")

Dashboard Title

# Generate synthetic data

np.random.seed(42)

num_batches = 100

df = pd.DataFrame({

"BatchID": [f"B{str(i).zfill(3)}" for i in range(num_batches)],

"MaterialLot": np.random.choice(['MLOT1', 'MLOT2', 'MLOT3', 'MLOT4'], size=num_batches),

"Machine": np.random.choice(['M1', 'M2', 'M3'], size=num_batches),

"Operator": np.random.choice(['O1', 'O2', 'O3', 'O4'], size=num_batches),

"Shift": np.random.choice(['Day', 'Night'], size=num_batches),

"Temperature": np.random.normal(loc=200, scale=10, size=num_batches).round(2),

"Pressure": np.random.normal(loc=5, scale=1.0, size=num_batches).round(2),

"Defective": np.random.choice([0, 1], size=num_batches, p=[0.85, 0.15])

})

Generates 100 batches with random values for:

- MaterialLot: which raw material lot was used

- Machine: which machine produced it

- Operator, Shift: human involvement

- Temperature, Pressure: process parameters

- Defective: randomly assigned (15% defective rate)

# Encode data

df_encoded = pd.get_dummies(df.drop(columns=["BatchID"]), drop_first=True)

X = df_encoded.drop(columns=["Defective"])

y = df_encoded["Defective"]

Encodes categorical variables (like Machine/Shift) into numbers

Split the data into:

- X: inputs (features)

- y: target (whether the batch is defective)

# Train/test split and model

X_train, X_test, y_train, y_test = train_test_split(X, y, test_size=0.2, random_state=42)

model = RandomForestClassifier(n_estimators=100, random_state=42)

model.fit(X_train, y_train)

Splits the data (80% training / 20% testing)

Trains a Random Forest classifier to predict defectiveness

# Add predictions to original dataframe

risk_scores = model.predict_proba(X)[:, 1] * 100

df["Defect Risk (%)"] = risk_scores.round(2)

Calculates the probability of defect for each batch

Adds it to the DataFrame as a new column

# Sidebar to select batch

selected_batch = st.sidebar.selectbox("Select Batch ID", df["BatchID"].unique())

# Display selected batch info

selected_info = df[df["BatchID"] == selected_batch]

st.subheader(f"Details for {selected_batch}")

st.write(selected_info.drop(columns=["Defective"]))

# Display defect risk

risk = selected_info["Defect Risk (%)"].values[0]

st.metric(label="Predicted Defect Risk", value=f"{risk:.2f} %", delta=None)

# Show table and chart

st.subheader("All Batch Risk Scores")

st.dataframe(df[["BatchID", "Defect Risk (%)"]].sort_values(by="Defect Risk (%)", ascending=False))

st.line_chart(df.set_index("BatchID")["Defect Risk (%)"])

Display Charts